SYSTEM ARCHITECTURE

The proposed system architecture consists of sensing, processing, machine learning and wireless communication modules integrated for real-time milk adulteration detection.  The milk sample is placed inside the sensing chamber where optical and conductivity analysis are performed simultaneously.

The AS7262 multispectral sensor captures spectral information of the milk sample at multiple wavelengths. Different adulterants alter the absorption and reflection characteristics of milk, producing unique spectral signatures.  The EC sensor measures the electrical conductivity of milk. Adulterants such as salt, detergent and urea change the ionic concentration of milk, resulting in conductivity variations.

- Both sensor outputs are collected by the ESP32 microcontroller, which acts as the central processing unit of the system. The ESP32 performs:

- Sensor interfacing

- Data acquisition

- Data preprocessing

- Feature extraction

- Machine learning classification

- A Logistic Regression machine learning model is used to classify the milk sample as:

- Pure OR

- Adulterated

- The classified result is transmitted wirelessly using:

- Bluetooth BLE OR

- Wi-Fi communication

- Real-time monitoring information

The complete system enables portable, low-cost and real-time milk adulteration detection suitable for dairy farms, milk collection centers and smart food quality monitoring applications.